Reflect & Assist

Kacey Morris

CST-452 Capstone Project Instructional Documentation

Grand Canyon University

Instructor: Professor Mark Reha

Revision: 1.0

Date: April 24, 2022

Instructional Documentation

Author: Kacey Morris

Review Date: April 24, 2022

Introduction

The Reflect & Assist is a Raspberry Pi kit with all materials and software included to create a Smart Mirror display. This display is a mobile mirror that projects information contained in modules alongside its reflection. The Reflect & Assist kit will include the Raspberry Pi, a monitor, plexiglass, links to required GitHub modules, as well as instructions on how to set up the Raspberry Pi, install all required dependencies, and assemble the mirror set up. As a kit, those interesting in building a customizable smart mirror will find all essential resources in a centralized location, making learning programming and embedded systems simpler and more accessible.

The kit will include 14 different display module options including an alarm clock, weather display, a daily Bible quote, and many others, as well as access to a website for managing the individual display. To encourage creativity and expansion, access to an extended list of potential modules and a template for personalized module creation has been included as well.

Materials

To establish the mirror, you will need:

- CanaKit Rasberry Pi 4 Model B Starter Kit

- Sceptre 24" Frameless Ultra Thin 1080P LED Monitor

- SupremeTech See-Through Two-Way Mirror (18x24 Inch, 0.04 Inch Thick Acrylic)

- Solid Foil Washi Tape Decorative Self Adhesive Masking Tape 15mm x 10 Meters (Gold)

- USB Keyboard

- USB Mouse

- Razor Blade

Physical Construction

- Follow the instructions in the Quick Start Guide to set up the Raspberry Pi. An online copy can be found here: https://www.canakit.com/quick-start/pi.

- Place the acrylic on a flat surface with the film still intact.

- Disconnect the monitor and place it screen down on the acrylic.

- Using a marker, trace the size of the monitor onto the acrylic film.

- Apply pressure to the acrylic and trace the marker lines with a razor blade.

- Repeat step 5 until a tactile divot is apparent in the acrylic.

- Place the larger side of the acrylic on the flat surface with the divot on the edge of the surface.

- Apply firm pressure to the larger side and bend the shorter side off the edge of the surface. It will snap off with a clean edge.

- Once this has been done for all extra acrylic, remove the film off both sides of the acrylic.

- Place the acrylic on the screen side of the monitor and use washi tape to secure the edges of the acrylic to the screen. The monitor should now be reflective.

- Reconnect the monitor to the Raspberry Pi.

Third Party Download

The smart mirror functionality relies on the installation of third-party software by MichMich on GitHub. Follow these installation instructions to establish the software: https://docs.magicmirror.builders/getting-started/installation.html#manual-installation. To stop the mirror, press Ctrl+Q. The following third-party additions will also require download.

Moon Phases - https://github.com/NolanKingdon/MMM-MoonPhase

Globe View - https://github.com/LukeSkywalker92/MMM-Globe

Daily Bible Verse - https://github.com/arthurgarzajr/MMM-DailyBibleVerse

Alarm Clock - https://github.com/fewieden/MMM-AlarmClock

Insults - https://github.com/mykle1/MMM-Insults

Sports - https://github.com/jclarke0000/MMM-MyScoreboard

Instructions for each of these downloads can be found in the README document connected to each repository.

Management Download / Hosting

The management site requires additional software for setup. To establish the server, language, and database, follow these instructions: https://raspberrytips.com/web-server-setup-on-raspberry-pi/#PHPMyAdmin. Once these have been established, run this SQL query to set up the management site database: https://github.com/KMorris63/SeniorProject1/blob/main/mirror.sql. The necessary management site files can then be downloaded here: https://github.com/KMorris63/SeniorProject1/tree/main/Mirror1. Download those files in /var/www/html on the Pi so they can be accessed on the Apache server.

From the terminal, type ifconfig and press enter. In the browser, enter [IP ADDRESS]/Mirror1/presentation/views/home.php

Additional Modules

Additional modules have been created by a variety of programmers. Here is a list of additional modules currently available: https://github.com/MichMich/MagicMirror/wiki/3rd-Party-Modules. If the desired module is not yet available, follow these instructions to create one:  https://docs.magicmirror.builders/development/introduction.html.

Amazon Links

Raspberry Pi - https://www.canakit.com/raspberry-pi-4-starter-kit.html

Two-Way Mirror - https://www.amazon.com/gp/product/B01CZ35XWY/ref=ppx_yo_dt_b_asin_title_o03_s01?ie=UTF8&psc=1

Monitor - https://www.amazon.com/gp/product/B0812BG24T/ref=ppx_yo_dt_b_asin_title_o05_s00?ie=UTF8&psc=1

Tape - https://www.amazon.com/Solid-Decorative-Adhesive-Masking-Meters/dp/B08H1H4B7P/ref=sr_1_3?crid=10EY2G2VSFDHS&keywords=gold%2Bwashi%2Btape&qid=1650774752&sprefix=gold%2Bwashi%2Btape%2Caps%2C115&sr=8-3&th=1

Keyboard - https://www.amazon.com/Manhattan-104-key-Keyboard-Built-Indicator/dp/B07RQVB3HQ/ref=sr_1_1_sspa?keywords=usb+keyboard&qid=1650776074&sprefix=usb+keyboa%2Caps%2C175&sr=8-1-spons&psc=1&spLa=ZW5jcnlwdGVkUXVhbGlmaWVyPUExODNKV0RUVjQyMVJJJmVuY3J5cHRlZElkPUEwNDM0MDY1MzU3QkRJRFRKNkdDTiZlbmNyeXB0ZWRBZElkPUEwMDQxOTQyTE1USjFVT1hRTjFFJndpZGdldE5hbWU9c3BfYXRmJmFjdGlvbj1jbGlja1JlZGlyZWN0JmRvTm90TG9nQ2xpY2s9dHJ1ZQ==

Mouse - https://www.amazon.com/AmazonBasics-3-Button-Wired-Computer-1-Pack/dp/B005EJH6RW/ref=sr_1_1_sspa?crid=3SEM81IRUC28G&keywords=usb+mouse&qid=1650776095&sprefix=usb+mous%2Caps%2C159&sr=8-1-spons&psc=1&spLa=ZW5jcnlwdGVkUXVhbGlmaWVyPUEyWFlTN1lNTE9KWEk3JmVuY3J5cHRlZElkPUEwNDQ1MTE2VFYyVlZCM1EzR1dWJmVuY3J5cHRlZEFkSWQ9QTA0ODI5ODYxVldOSDdUNE1KNDdOJndpZGdldE5hbWU9c3BfYXRmJmFjdGlvbj1jbGlja1JlZGlyZWN0JmRvTm90TG9nQ2xpY2s9dHJ1ZQ==